| Attribute | T/E | Example | Kind | Type |
| --- | --- | --- | --- | --- |
| Attribute description for the specified example |  |  |  |  |
| ‘DELAYED | - | s1’DELAYED (5 NS) | SIGNAL | As s1 |
| A copy of s1, but delayed by 5 NS. If no parameter or 0, delayed by delta. Equivalent to TRANSPORT delay of s1. |  |  |  |  |
| ‘STABLE | EV | s1’STABLE (5 NS) | SIGNAL | BOOLEAN |
| A signal that is TRUE if s1 has not changed in the last 5 NS. If no parameter or 0, the resulting signal is TRUE if s1 has not changed in the current simulation time. |  |  |  |  |
| ‘EVENT | EV | S1’EVENT | VALUE | BOOLEAN |
| In a simulation cycle, if s1 changes, this attribute becomes TRUE. |  |  |  |  |
| ‘LAST_EVENT | EV | s1’LAST_VALUE | VALUE | TIME |
| The amount of time since the last value change on s1. If s1’EVENT is TRUE, the value of s1’LAST_VALUE is 0. |  |  |  |  |
| ‘LAST_VALUE | EV | s1’LAST_VALUE | VALUE | As s1 |
| The value of s1 before the most recent event occurred on this signal. |  |  |  |  |
| ‘QUIET | TR | s1’QUIET (5 NS) | SIGNAL | BOOLEAN |
| A signal that ir TRUE if no transaction has been placed on s1 in the last 5 NS. If no parameter or 0, the current simulation cycle is assumed. |  |  |  |  |
| ‘ACTIVE | TR | s1’ACTIVE | VALUE | BOOLEAN |
| If s1 has had a transaction in the current simulation cycle, s1’ACTIVE will be TRUE for this simulation cycle, for delta time. |  |  |  |  |
| ‘LAST_ACTIVE | TR | s1’LAST_ACTIVE | VALUE | TIME |
| The amount of time since the last transaction occurred on s1. If s1’ACTIVE is TRUE, s1’LAST_ACTIVE is 0. |  |  |  |  |
| ‘TRANSACTION | TR | s1’TRANACTION | SIGNAL | BIT |
| A signal that toggles each time a transaction occurs on s1. Initial value of this attribute is not defined. |  |  |  |  |
| ‘DRIVING | - | s1’DRIVING | VALUE | BOOLEAN |
| If s1is being driven in a process, s1’DRIVING is TRUE in the same process. |  |  |  |  |
| ‘DRIVING_VALUE | - | s1’DRIVING_VALUE | VALUE | As s1 |
| The driving value of s1 from within the process this attribute is being applied. |  |  |  |  |